ChatBox 40-XT Plus Quick Reference Guide

This guide is to be used as a quick reference but is not intended to replace the application guide that came with your ChatBox 40-XT Plus.

Choose an Overlay:

The ChatBox 40-XT Plus allows for 10 different overlays.

- Use the Up or Down arrow buttons in the Control Panel Area on the front of the ChatBox 40-XT to choose Overlays

- Remove the keyguard and replace the paper overlay with one that corresponds to the stored vocabulary.

Navigating Menus:

Choose Menu to open the listing of menus

Choose Prev or Next to navigate through the menu options

Choose Select to activate the option

Choose Menu to close the listing of menus

The programming menus use their own instructions, read the screen for instructions.

Changing the keyboard layout:

The ChatBox 40-XT comes with a 10 and a 40-location keyboard, overlays and keyguard.

Both physical and settings must be changed.

Hardware:

- remove the keyguard, overlays and keyboard

- replace the keyboard with the appropriate size (making sure the holes for the lights are in the upper left corners).

- overlays need to be created either with the Boardmaker program from Mayer Johnson or by hand and inserted into the opening on the keyboard

- replace the keyguard with the different size

Settings:

- choose Menu from the Control Area on the front of the ChatBox 40-XT

- tap the Next button until you see Selection Technique Options

- choose Select

- choose Select again

- tap Next to change the number of keys

- choose Menu to close the menu

Record a message on a key:

- choose Menu

- choose Select

- Press a key (or sequence of keys)

- Push and hold Select while speaking into the microphone (release immediately after message has been spoken)

- If  Text Messages are On, you will be asked to type a message that will be displayed

- continue to program the next key or choose Menu to Exit

The alphabet for programming messages has a pre-defined layout.  The alphabet layout can be found by laying the clear overlay protector  with the alphabet layout that came with the system over the existing overlay.

Store a Synthesized (computer speaks) Message on a key:

1.  choose Menu

2.  choose Next

3.  choose Select

4.  press a key (or a sequence if keys) where you want to store text

5.  choose Select

6.  type the message to be spoken

7.  choose Select

8.  continue to store another message or press Menu to Exit

The alphabet for programming messages has a pre-defined layout.  The alphabet layout can be found by laying the clear overlay protector with the alphabet layout that came with the system over the existing overlay.

Program a Function on a key:

The ChatBox 40-XT offers a number of functions that can be added to your overlay.

- choose Menu

- choose Next until you see Program Function

- choose Select

- press the key(or sequence of keys) where you want to program a function

- choose Select

- choose Next to step through the functions

- choose Select to activate the function

- choose Menu to Exit

Clear Display – clears the display of all text

Delete Last Selection – clears the last selection only

Speak Display – speak what is listed on the display

Speak Compiled Message – speaks recorded messages previously chosen

Left Arrow – moves the cursor left through the text

Right Arrow – moves the cursor right through the text

Spelling Mode – moves the overlay into the pre-defined spelling layout, choosing Exit in lower right corner exits Spelling Mode

Add S – deletes a character, adds an s and a space

Add ED – deletes a character, adds ed and a space

Add ING – deletes a character, adds ing and a space

Shift – shifts the next character, selecting Shift twice locks the  shift On.

Change Overlay – provides access  for changing overlays from the keyboard

Toggle Speech On/Off – provides ability to turn the speech on/off from the keyboard

Erase a key (or sequence)

- choose Menu

- choose Next twice

- choose Select

- press the key (or sequence) to be erased

- choose Select

- choose Select again to confirm you want to erase

- continue to erase or choose Menu to Exit

Erase an entire overlay:

- choose Menu

- choose Next twice

- choose Select

- choose the Down Overlay arrow button

- press Select

Note:  DO NOT choose Select twice at this point or the entire device contents will be erased

- choose Menu to Exit

Erase the entire device contents:

- choose Menu

- choose Next twice

- choose Select

- choose the Down Overlay arrow button

- press Select twice

- choose Menu to Exit

Lock control panel:

The ChatBox 40-XT has the option to Lock the Control Panel.  The options are to lock just the Menu button, just the Overlay buttons or both.

- choose Menu

- choose Next until you see “User Feedback Options”

- choose Select

- choose Next until you see “Lock Controls”

- choose Select

- choose Next to lock the menus, overlays or both

- choose Menu to exit

Pushing and holding the Menu or Overlay button for 10 seconds will enable access to the controls again.

Saltillo Corporation

2143 TR 112

Millersburg, OH  44654

Phone: (330)674-6722 – (800)382-8622